# Radix

Radix is a Latin word for "root". Root can be considered a synonym for base in the arithmetical sense.

In mathematical numeral systems, the radix or base is the number of unique digits, including zero, used to represent numbers in a positional numeral system. For example, for the decimal system (the most common system in use today) the radix is ten, because it uses the ten digits from 0 through 9.

In any standard positional numeral system, a number is conventionally written as (x)y with x as the string of digits and y as its base, although for base ten the subscript is usually assumed (and omitted, together with the pair of parentheses), as it is the most common way to express value. For example, (100)dec = 100 (in the decimal system) represents the number one hundred, while (100)2 (in the binary system with base 2) represents the number four.

## In numeral systems

In the system with radix 13, for example, a string of digits such as 398 denotes the number 3 × 132 + 9 × 131 + 8 × 130.

More generally, in a system with radix b (b > 1), a string of digits d1 … dn denotes the number d1bn−1 + d2bn−2 + … + dnb0, where 0 ≤ di < b.

Commonly used numeral systems include:

| Base/Radix | Name | Description |
| --- | --- | --- |
| 2 | binary numeral system | used internally by nearly all computers, is base two. The two digits are "0" and "1", expressed from switches displaying OFF and ON respectively. Used in most electric counters. |
| 8 | octal system | is occasionally used in computing. The eight digits are "0–7" and represent 3 bits (23). |
| 10 | decimal system | the most used system of numbers in the world, is used in arithmetic. Its ten digits are "0–9". Used in most mechanical counters. |
| 12 | duodecimal (dozenal) system | is sometimes advocated due to divisibility by 2, 3, 4 and 6. It was traditionally used as part of quantities expressed in dozens and grosses. |
| 16 | hexadecimal system | is often used in computing as a compacter representation of binary (1 hex digit per 4 bits). The sixteen digits are "0–9" followed by "A–F" or "a–f". |
| 20 | vigesimal | traditional numeral system in several cultures, still used by some for counting. |
| 60 | sexagesimal system | originated in ancient Sumer and passed to the Babylonians.[2] Used today as the basis of our modern circular coordinate system (degrees, minutes, and seconds) and time measuring (hours, minutes, and seconds). |

The octal and hexadecimal systems are often used in computing because of their ease as shorthand for binary. Every hexadecimal digit corresponds to a sequence of four binary digits, since sixteen is the fourth power of two; for example, hexadecimal 7816 is binary 11110002. A similar relationship holds between every octal digit and every possible sequence of three binary digits, since eight is the cube of two.

Radices are usually natural numbers. However, other positional systems are possible, e.g. golden ratio base (whose radix is a non-integer algebraic number),[3] and negative base (whose radix is negative).

BOOLEAN ALGEBRA

The digital circuitry in digital computers and other digital systems is designed, and its behavior is analyzed, with the use of a mathematical discipline known as Boolean algebra. The name is in honor of an English mathematician George Boole, who proposed the basic principles of this algebra in 1854 in his treatise

Boolean algebra turns out to be a convenient tool in two areas:

- Analysis: It is an economical way of describing the function of digital circuitry.

- Design: Given a desired function, Boolean algebra can be applied to develop a simplified implementation of that function.

As with any algebra, Boolean algebra makes use of variables and operations.

In this case, the variables and operations are logical variables and operations. Thus, a

variable may take on the value 1 (TRUE) or 0 (FALSE). The basic logical operations are AND, OR, and NOT, which are symbolically represented by dot, plus sign, andoverbar2:

A AND B = A . B

A OR B = A + B

NOT A = A’

The operation AND yields true (binary value 1) if and only if both of its operands are true. The operation OR yields true if either or both of its operands are true. The unary operation NOT inverts the value of its operand. For example, consider the equation

D = A + (B . C)

D is equal to 1 if A is 1 or if both B = 0 and C = 1. Otherwise D is equal to 0.

Several points concerning the notation are needed. In the absence of parentheses, the AND operation takes precedence over the OR operation. Also, when no ambiguity will occur, the AND operation is represented by simple concatenation instead of the dot operator. Thus,

A + B . C = A + (B . C) = A + BC

all mean: Take the AND of B and C; then take the OR of the result and A. Table 11.1a defines the basic logical operations in a form known as a truth table, which lists the value of an operation for every possible combination of values of operands. The table also lists three other useful operators: XOR, NAND, and NOR. The exclusive-or (XOR) of two logical operands is 1 if and only if exactly one of the operands has the value 1. The NAND function is the complement (NOT) of the AND function, and the NOR is the complement of OR:

A NAND B = NOT (A AND B) = AB

A NOR B = NOT (A OR B) = A + B

As we shall see, these three new operations can be useful in implementing certain digital circuits.

The logical operations, with the exception of NOT, can be generalized to more than two variables, as shown in Table 11.1b. Table 11.2 summarizes key identities of Boolean algebra. The equations have been arranged in two columns to show the complementary, or dual, nature of the AND and OR operations. There are two classes of identities: basic rules (or postulates), which are stated without proof, and other identities that can be derived from the basic postulates. The postulates define the way in which Boolean expressions are interpreted. One of the two distributive laws is worth noting because it differs from what we would find in ordinary algebra:

A + (B . C) = (A + B) . (A + C)

Table 11.1 Boolean Operators

(a) Boolean Operators of Two Input Variables

Refer to: (9th Edition) (William Stallings Books on Computer and Data Communications) William Stallings-Computer Organization and Architecture-Prentice Hall (2012) (pages 366-67)

LOGIC GATES

The fundamental building block of all digital logic circuits is the gate. Logical functions are implemented by the interconnection of gates.