This summary will talk about CPU architecture, we will look at types of CPU architecture, data path of a CPU, the fetch-decode-execute cycle what it is and how it work, microinstruction sequencing, implementation of control unit calculation of MIPS parameters

TYPES OF CPU ARCHITECTURE

Reduced Instruction Set Architecture (RISC), emerged around early 80s.  Designers re-evaluating the current ISAs of the era found that ISAs had extensive instructions that

were complex. Complex Instruction Set Architecture (CISC) need only 20% of the instructions that were used most of the time.

- COMPLEX INSTRUCTION SET ARCHITECTURE

  - Memory in those days was expensive because there weren’t alot computers as compared to today.

  - bigger programs mean more storage mean more money so that’s when this architecture came to effective use

  - Hence, they needed to reduce the number of instructions per program

  - Number of instructions are reduced by having multiple operations within a single instruction.

  - Multiple operations lead to many different kinds of instructions that access memory.

  - In turn making instruction length variable and fetch-decode execute time unpredictable therefore making it more complex.

  - Thus, hardware handles the complexity, an example is the x86 ISA.

- REDUCED INSTRUCTION SET ARCHITECTURE

- Original idea to reduce the ISA.

- Provide minimal set of instructions that could carry out all essential operations.

- Instruction complexity is reduced by.

- Having few simple instructions that are the same length.

- Allowed memory access only with explicit load and store instructions.

- Hence each instruction performs less work but instruction execution time among different instructions is consistent.

- The complexity that is removed from ISA is moved into the domain of the assembly programmer/compiler.

- Examples include LC3, MIPS, PowerPC (IBM), SPARC (Sun) just to mention a few.

DATA PATH OF A CPU

The datapath is the part of the central processing unit (CPU) that does the actual computations.

FETCH-DECODE-EXECUTE CYCLE

The Von Neumann architecture uses a single processor which follows a linear sequence of fetch-decode-execute. In order to do this, the processor has to use some special registers, which are discrete memory locations with special purposes attached. So all what this does is connect registers.the registers are:

  - the Program Counter

  - Current Instruction Register

  - Memory Address Register

  - Memory Data Register

  - Index Register

  - Accumulator

CALCULATION OF MIPS

for accessing memory.

Millions of Instructions Per Second(MIPS) Parameters

A common performance measure for the CPU rate. MIPS are machine dependent. Varies from architecture to architecture.

Conditions for using MIPS:

•	The Same Program is being used.

•	The same ISA is used.

•	The Same Compiler is used.

Calculating MIPS:

MIPS = instruction count/ Execution Time × 106

But, execution time = IC×CPI / Clock rate

...      MIPS = 	f / (CPI × 106)

Given by the formula

IMPLIMITATION OF CONTROL UNIT

Is a part of the CPU, it controls the flow of data between the processor and memory and peripherals. Hence, its function is to generate relevant timing and control signals to all operations in the computer. It has two major designs

- Hardwared control unit

- Micro-programmed control unit