Fault Isolation Process

Fault isolation refers to the identification and pinpointing of source fault element in a network.

In this process, the exact location of the root fault is determined and the element is isolated for repair

Steps in Fault Isolation Process

- Detecting a problem

- Providing a clear understanding of the problem (also known as a problem statement)

- Obtaining information

- Involves collecting necessary information about a network

- Analysing and classifying information

- Involves examining, evaluating and grouping of collected information for easier fault isolation

- Fault isolation

- Involves exact identification and elimination of fault root from the network

- Problem solving and verification

- After isolating, it is easier to apply a solution as required, either permanent or temporary

- Documentation of problem and solution

- Keeping detailed records of the problem and the solutions involved

Importance of Documentation

- Progress tracking

- Allows professionals to know how far they are in resolving a problem

- It helps avoid redundant effort and repetition as they know what has been tried and not, as well as what worked and what did not

- Keeps track of everyone’s contributions during the process thus enhancing progress and team work

- Knowledge transfer

- Since troubleshooting is mostly team effort, it’s easier to share insight of what is happening with colleagues and higher authorities

- Makes it easier when collaborating with external support to find solution as they can easily follow the records

- It provides a start point for future members and makes it easier for them to understand and resolve a problem

- Reference material

- Provides an available solution or a start point for when a problem will recur in future

- Helps avoid intellectual property theft (plagiarism)

Troubleshooting Tools

- These are utilities used in diagnosing and resolving network issues(errors).

Command-line Tools

- These are tools executed via command prompt as text

- Ping

- Tests connectivity between two devices by measuring the time a message takes to reach destination computer from sender

- Used to check availability of a device over a network. A negative ping request indicates a problem between source device and destination (diagnosing connectivity)

- Traceroute

- Determine the path data packets take across an IP network

- Used to identify where there is delay or losses of packets in a network traffic (diagnosing router)

- Provides a detailed trace of data path but can be slow

- Nslookup

- A Domain Name System that provides IP address/domain name of host

- Used to identify DNS issues and domain resolution (diagnosing DNS)

- It is quick, easy and working on most OS

- Netstat

- Shows status in respect to network connectivity and number of active network connections

- Used to monitor running services on a system and network traffic (diagnosing network configuration)

- Provides detailed information but is complex

Importance/advantages/roles of troubleshooting tools

- Ensuring reliability

- Ensuring security

https://www.pcmag.com/encyclopedia/term/fault-isolation